VOICE agent
Text for an Invitation to Tender 
Stand-alone unit for condition monitoring and remote alarming. Mounting on DIN rail and standard section for cabinets. Wide-range power supply of 100-240VAC or 18-70VDC (24VDC). Low voltage for signal inputs internally generated. Total of 16 digital inputs, up to 8 of which can be configured as analogue inputs. 4 outputs, potential-free relay contacts. A display to show the operating status and alarms. Control wheel for an easy configuration of subscriber data and device settings. Integrated controlled battery charging for connecting an external battery. Monitoring the supply voltage, battery voltage and the telephone network connection. Built-in landline or GSM modem. Programming without any special software via web browser. Possibility of remote maintenance via the LAN or telephone network. Sending voice messages, text messages or e-mail. Ability to send cyclic test alarms.Eight on-call groups with up to 30 users are configurable for the alarming process. The alarm reporting process takes place in accordance with DIN 19235. Each user included on the alarm list is able to acknowledge the alarm by entering a code (DTMF tones) and thus, interrupt the alarming of further users on the list. Eight timer channels can be used for the scheduling of the 8 on-call groups. 

Proposed Brand
Product:
VOICE agent
Manufacturer
APS systems AG, www.aps-systems.ch:

Alternative offer:
Product:
________________________________________

Manufacturer:
________________________________________

Specifications:

Supply voltage AC version:
230VAC (100-240VAC/ 47-62-3Hz)
Supply voltage DC version:
24VDC (18-70VDC)
Network:
Ethernet 10BaseT IEE 802,3 10MB

IP address:
DHCP or fixed
Power consumption:
max. 10W
Protection rating:
IP 20

Weight:
650g

Housing:
Steel, powder-coated 

Temperature range:
0°C to +40°C

Dimensions:
175x120x68mm
Output relays:
4, max. 6A AC1

Digital inputs 
up to 16

Analogue inputs:
up to 8, 0-10V (0-20mA with shunt)

Display:
3 lines LCD

Name:
VOICE agent landline 110-230VAC
832.200.000


VOICE agent landline 24VDC
832.200.100


VOICE agent GSM 110-230VAC
832.210.000


VOICE agent GSM 24VDC
832.210.100

Accessories:
Battery 1.2Ah
192-000

Smoke alarm
901-0103


Magnetic door contact  
901-0109


Water/oil sensor 5mm with 5m cable
SWM5


Humidity/temperature sensor 0-100%rF -30 to 70°C without a display
AFT-U


Humidity/temperature sensor 0-100%rF -0 to 50°C without a display
RHT-01


Humidity/temperature sensor 0-100%rF -0 to 50°C with a display
KLH-100-N

Vendor:
APS systems AG


Neumatt 4

CH-4626 Niederbuchsiten

Tel. 
+41 (0)62 517 88 33
www.aps-systems.ch
E-mail
systems@aps-systems.ch
